INDUSTRIAL TRAINING REPORT

On

“VISTA — Vision-based Intelligent Stevenson Thermometer Analyzer”

Submitted in partial fulfillment of the requirements for the award of the degree of

BACHELOR OF TECHNOLOGY

in

ELECTRONICS AND COMMUNICATION ENGINEERING

By

ABHIJEET RASTOGI

and

MOHAK JOSHI

Under the Guidance of

____________________________

India Meteorological Department (IMD), New Delhi

2026

# ABSTRACT

This industrial training project presents a Raspberry Pi-based digital image-processing system for automated observation of a thermometer installed inside a Stevenson Screen. The primary objective is to reduce dependence on manual visual reading by acquiring thermometer images through a camera, processing the relevant image region, estimating the mercury level, converting its pixel position into temperature, displaying the result, and recording the measurement digitally.

The implemented system uses Picamera2 for image acquisition and OpenCV and NumPy for image processing. Five consecutive frames are averaged to reduce short-term image noise. The frame is then processed using bilateral filtering, grayscale conversion, CLAHE-based contrast enhancement, and sharpening. A narrow Region of Interest (ROI) around the thermometer column is extracted. A stored reference ROI is compared with the live ROI using absolute image difference. The resulting difference image is reduced to a one-dimensional vertical profile, which is smoothed and analyzed using an adaptive threshold based on background intensity and image contrast.

The mercury position is detected by searching for a sustained threshold crossing across multiple consecutive rows. A row-correction framework is also included. The corrected pixel position is converted to temperature using a two-point linear calibration. The calculated temperature is shown on a live graphical interface and recorded in a CSV file at a 30-second interval.

The supplied implementation represents a working prototype. A formal numerical accuracy percentage is not claimed in this report because the source project does not contain a completed statistical validation study. Experimental comparison with a calibrated reference thermometer is recommended for future work.

# TABLE OF CONTENTS

| Chapter / Section | Page No. |
| --- | --- |
| Certificate | i |
| Declaration | ii |
| Acknowledgement | iii |
| Abstract | iv |
| Chapter 1 – Company Profile | 1 |
| Chapter 2 – Work Undertaken | 4 |
| Chapter 3 – Project Carried Out | 6 |
| Chapter 4 – Learning Outcomes | 15 |
| Chapter 5 – Conclusion | 18 |
| References | 19 |

# LIST OF FIGURES

| Figure No. | Title |
| --- | --- |
| 1.1 | Overall system architecture |
| 1.2 | Image-processing pipeline |
| 1.3 | Mercury detection and temperature conversion flow |
| 1.4 | Graphical user interface |

# LIST OF TABLES

| Table No. | Title |
| --- | --- |
| 1.1 | Hardware components |
| 1.2 | Software libraries and roles |
| 1.3 | Key configuration parameters |
| 1.4 | Challenges, observations and solutions |

# CHAPTER 1

# COMPANY PROFILE

## 1.1 India Meteorological Department (IMD)

The India Meteorological Department (IMD) is India's national meteorological service and operates under the Ministry of Earth Sciences, Government of India. IMD is responsible for meteorological observations, weather forecasting, climatological services, warnings, and several specialized atmospheric and environmental services.

Meteorological observations depend on reliable instruments, data acquisition, processing, and interpretation. The use of digital systems and automation can support the collection of consistent observations and reduce dependence on manual transcription.

## 1.2 Relevance of the Project to IMD

The project developed during the training addresses one aspect of meteorological observation: automatic interpretation of a thermometer reading. By using a camera and image processing, the system demonstrates how conventional visual instruments can be interfaced with digital computing hardware for automated observation and logging.

## 1.3 Technical Environment

The project combines embedded computing, camera interfacing, image processing, GPIO control, graphical user-interface development, and data logging. The Raspberry Pi acts as the central compute platform, while the camera provides the image input and the software performs the measurement pipeline.

# CHAPTER 2

# WORK UNDERTAKEN

## 2.1 Role During Training

The training work involved the development and refinement of a digital image-processing prototype for thermometer observation. The work required understanding the physical observation setup, configuring a Raspberry Pi camera, selecting a suitable ROI, developing image-processing operations, tuning detection thresholds, implementing temperature calibration, creating a GUI, and implementing automatic data logging.

## 2.2 Tools and Technologies Used

| Tool / Technology | Application in Project |
| --- | --- |
| Raspberry Pi 4B | Embedded computing and execution platform |
| Picamera2 | Camera configuration and frame acquisition |
| OpenCV | Image filtering, enhancement, difference processing and visualization |
| NumPy | Frame averaging and numerical profile calculations |
| Python | Main application programming language |
| RPi.GPIO | LED control through GPIO |
| Tkinter | Graphical user interface |
| CSV / datetime | Measurement logging |
| threading | Concurrent camera processing and logging |

## 2.3 Skills Acquired

Raspberry Pi camera configuration and image acquisition

Digital image preprocessing using OpenCV

ROI-based image analysis

Difference-image and one-dimensional profile analysis

Adaptive threshold design

Pixel-to-temperature calibration

GPIO-based hardware control

Python GUI development using Tkinter

CSV-based automated data logging

Debugging and parameter tuning for an embedded vision application

CHAPTER 3

# PROJECT – VISTA — Vision-based Intelligent Stevenson Thermometer Analyzer

## 3.1 Project Introduction

The project aims to automate thermometer observation in a Stevenson Screen using a camera and image-processing algorithm. The system captures the thermometer image, focuses on a narrow ROI, compares the live ROI with a stored reference, detects the mercury level, calculates temperature, and logs the result.

## 3.2 Project Objectives

Automate the observation of the thermometer level.

Develop a robust image-processing pipeline for the narrow thermometer ROI.

Reduce image noise and improve local contrast.

Use reference-image comparison to highlight relevant changes.

Detect the mercury level using a row-wise profile and adaptive threshold.

Convert the detected pixel position into a temperature reading.

Provide real-time visual feedback through a GUI.

Maintain a timestamped digital record of measurements.

## 3.3 Problem Statement

Manual observation of a thermometer requires a person to visually inspect the instrument and record the reading. This process is not ideal for continuous automated monitoring because it requires human presence and can introduce reading or transcription errors. The project therefore investigates a camera-based approach in which the position of the mercury column is inferred from image data and converted into a digital temperature value.

## 3.4 Rationale

A camera-based approach makes it possible to digitize a conventional visual instrument without replacing the thermometer itself. The Raspberry Pi provides a compact platform capable of camera acquisition, image processing, display, hardware control, and data logging in a single system.

## 3.5 System Architecture / Flow

The implemented system can be represented as:

Camera → Frame Averaging → Image Preprocessing → ROI Extraction → Reference Comparison → Absolute Difference → 1D Vertical Profile → Adaptive Threshold → Mercury Detection → Temperature Calibration → Display → CSV Logging

Figure 1.2: Detailed image-processing flow based on the implemented project.

## 3.6 Hardware Components

| Component | Specification / Use |
| --- | --- |
| Raspberry Pi 4B | Main processing platform |
| Camera module | Image acquisition through Picamera2 |
| Stevenson Screen | Enclosure for thermometer observation |
| Thermometer | Source of temperature indication |
| White LED | Controlled illumination; GPIO BCM 18 |
| Display | Live preview and GUI |

## 3.7 Software and Libraries

| Library / Module | Role |
| --- | --- |
| cv2 | Filtering, enhancement, difference image, display |
| numpy | Frame averaging and profile operations |
| picamera2 | Camera acquisition |
| libcamera | Camera transform/configuration |
| RPi.GPIO | GPIO and LED control |
| tkinter | GUI |
| csv | Data storage |
| datetime | Timestamp generation |
| threading | Concurrent execution |
| os / time | File management and timing |

## 3.8 Implementation Details

### 3.8.1 Camera Initialization

The camera is initialized using Picamera2 with a 700 × 640 RGB888 video configuration. The configured transform uses no horizontal or vertical flip in the supplied code. The camera is started before the processing loop begins.

### 3.8.2 Frame Averaging

For every processing iteration, five frames are captured. Each frame is converted to a floating-point array, the five arrays are averaged, and the result is converted back to an 8-bit image. This reduces short-term random variation before subsequent processing.

### 3.8.3 Image Preprocessing

The preprocessing stage applies a bilateral filter. In Black_White mode, the frame is converted to grayscale, CLAHE is used for local contrast enhancement, and a sharpening kernel is applied. The resulting grayscale image is converted back to BGR for compatibility with the display pipeline.

### 3.8.4 Region of Interest

The current ROI is defined by ROI_X=323, ROI_Y=1, ROI_W=9, and ROI_H=620. This narrow vertical ROI concentrates the algorithm on the thermometer column and reduces unnecessary processing of the surrounding Stevenson Screen.

### 3.8.5 Reference Image

The user can capture the current ROI as a reference image through the GUI. The reference is stored as REFERENCE.png. During normal processing, the reference is loaded and compared with the current live ROI.

### 3.8.6 Reference and Live Image Preparation

The reference and live ROI are converted to grayscale when necessary and Gaussian blurred using a 5 × 5 kernel. Blurring reduces small high-frequency variations before the difference operation.

### 3.8.7 Absolute Difference

The core comparison operation is an absolute pixel-wise difference between the reference and live grayscale images. This produces a difference image that emphasizes changes between the reference condition and the current thermometer image.

### 3.8.8 One-Dimensional Vertical Profile

The mean difference value is calculated for every image row. The resulting one-dimensional profile represents how strongly each vertical position differs from the reference. The profile is smoothed using a Gaussian filter.

### 3.8.9 Adaptive Threshold

The background level is estimated using the mean of the first 20 profile values. A peak estimate is obtained from the mean of the highest five profile values. Contrast is calculated as peak minus background. The calculate_factor() function then selects a threshold factor according to the background and contrast ranges.

### 3.8.10 Mercury Level Detection

The profile is scanned from top to bottom. The implementation requires seven consecutive rows to be at or above the threshold before accepting a mercury row. This consecutive-row condition is intended to reduce false detections caused by isolated noise. If no valid crossing is detected, the maximum profile location is used as a fallback.

### 3.8.11 Row Correction

The detected mercury row is passed to row_factor(), which interpolates correction values from the ROW_FACTOR table. In the supplied configuration, all correction values are zero, so the current table does not change the detected row.

### 3.8.12 Display and Logging

The calculated temperature is drawn on the live preview. A separate logging thread writes the date, time, and temperature to temperature_log.csv every 30 seconds.

## 3.9 Calibration

The supplied calibration settings are shown below.

| Parameter | Current value |
| --- | --- |
| PIXEL_AT_5C | 505 |
| PIXEL_AT_45C | 63 |
| TEMP_LOW | 12 °C |
| TEMP_HIGH | 45 °C |

The code uses a linear mapping of the form:

Temperature = TEMP_LOW + ((PIXEL_AT_5C − Mercury_Row) / (PIXEL_AT_5C − PIXEL_AT_45C)) × (TEMP_HIGH − TEMP_LOW)

An important implementation note is that the variable name PIXEL_AT_5C suggests a 5 °C calibration point, while TEMP_LOW is currently set to 12 °C. The report therefore treats these as the configured calibration endpoints rather than asserting that the system has been experimentally calibrated from 5 °C to 45 °C.

## 3.10 Adaptive Threshold Factor

The adaptive threshold factor is selected using background intensity and contrast. The supplied code defines three broad background categories and four contrast conditions within each category. This allows the final threshold to vary according to the observed scene.

## 3.11 Graphical User Interface

| Control | Function |
| --- | --- |
| LED ON | Turns on the white LED and selects White RGB mode. |
| BLACK & WHITE | Selects Black_White mode and disables automatic exposure/white balance. |
| LED OFF | Turns off the white LED. |
| TAKE REFERENCE PHOTO | Captures and stores the current ROI as the reference. |
| SCREENSHOT | Stores the current ROI with a user-entered temperature label. |
| QUIT | Stops processing, turns off the LED, cleans GPIO and closes the GUI. |

## 3.12 Data Logging

The logging function creates temperature_log.csv when necessary, writes the header Date, Time, Temperature (°C), and appends the current non-zero temperature. The logging interval is 30 seconds.

Example format:

Date, Time, Temperature (°C)
2026-08-09, 09:30:00, 30.4
2026-08-09, 10:00:00, 30.6

## 3.13 Results and Observations

The supplied implementation demonstrates the complete flow from image acquisition to digital temperature output. The system includes a continuous camera loop, preprocessing, reference comparison, mercury-level detection, temperature conversion, live visualization, and periodic logging.

The project source code does not contain a formal experimental accuracy study. Therefore, no unsupported numerical accuracy claim is included in this report. Accuracy should be determined by comparing the system output against a calibrated reference thermometer over a suitable set of temperature points and environmental conditions.

## 3.14 Challenges Faced and Solutions

| Challenge / Observation | Implemented approach / Solution |
| --- | --- |
| Frame-to-frame image variation | Five-frame averaging before processing. |
| Image noise | Bilateral and Gaussian filtering. |
| Low/local contrast | CLAHE and sharpening. |
| Large image area | Narrow ROI around thermometer column. |
| Changing background/contrast | Adaptive threshold factor based on measured background and contrast. |
| False detections from isolated peaks | Seven consecutive rows required for mercury detection. |
| Small positional bias | ROW_FACTOR interpolation framework. |
| Need for automated records | CSV logging thread with 30-second interval. |
| Different illumination modes | GUI controls for LED and Black_White/White RGB operation. |

## 3.15 Limitations

The numerical accuracy of the system has not been formally established.

The calibration variable naming and configured temperature endpoint require clarification before final validation.

The current row correction offsets are zero.

The narrow ROI requires stable camera placement.

The main detection path depends on a valid reference image.

Adaptive threshold ranges require empirical tuning.

The fallback maximum-profile method can return a position even when a sustained threshold crossing is not found.

## 3.16 Future Scope

Perform multi-point calibration against a certified reference thermometer.

Quantify MAE, maximum error, repeatability, and detection failure rate.

Test robustness under different lighting and camera positions.

Record background, contrast, threshold, mercury row, and temperature for offline analysis.

Add confidence flags for threshold-based versus fallback detections.

Automate reference-image quality checking.

Optimize CPU usage and quantify processing latency on Raspberry Pi 4B.

Move calibration and threshold settings into a separate configuration file.

Develop long-term monitoring and visualization of the logged temperature data.

# CHAPTER 4

# LEARNING OUTCOMES

## 4.1 Technical Skills Gained

Embedded Linux and Raspberry Pi application development

Camera interfacing using Picamera2

OpenCV-based image filtering and enhancement

NumPy-based numerical image analysis

ROI selection and image differencing

One-dimensional signal/profile extraction from images

Adaptive threshold design and parameter tuning

Temperature calibration from image coordinates

GPIO control for illumination

Tkinter GUI development

Multithreading for concurrent processing and logging

CSV-based data acquisition and storage

## 4.2 Problem-Solving Skills

The project required iterative debugging and tuning because image-processing performance depends on illumination, camera positioning, contrast, and the physical appearance of the thermometer. The development process involved identifying the useful image region, reducing noise, selecting appropriate profile statistics, designing an adaptive threshold, and introducing a row-correction framework.

## 4.3 Understanding of Digital Image Processing

The project provided practical experience in converting a visual measurement problem into a sequence of numerical image operations. In particular, it demonstrated how filtering, contrast enhancement, subtraction, spatial averaging, thresholding, and coordinate calibration can be combined to extract a physical measurement from an image.

## 4.4 Embedded-System Exposure

Because the application executes on a Raspberry Pi, the project also provided exposure to resource-constrained computing, hardware interfaces, camera configuration, GPIO control, real-time display, and periodic data logging.

## 4.5 Industry Exposure

The work demonstrated the importance of translating a physical measurement requirement into a repeatable software pipeline. It also highlighted the need for calibration, validation, error analysis, documentation, and maintainability when developing an engineering prototype.

## 4.6 Gaps Identified and Next Steps

| Area | Gap / Observation | Next Step |
| --- | --- | --- |
| Accuracy validation | No formal accuracy study in current code. | Conduct reference-thermometer experiments. |
| Calibration | Endpoint naming/configuration needs clarification. | Recalibrate and document exact endpoints. |
| Robustness | Thresholds are empirically defined. | Test across lighting and temperature conditions. |
| Data analysis | Current CSV stores only final measurements. | Log intermediate detection features. |
| Detection confidence | Fallback detection is not explicitly flagged. | Add confidence/status output. |

# CHAPTER 5

# CONCLUSION

The VISTA — Vision-based Intelligent Stevenson Thermometer Analyzer project demonstrates a practical approach to automating thermometer observation using a Raspberry Pi and camera. The implemented software integrates image acquisition, multi-frame averaging, preprocessing, ROI extraction, reference-image comparison, difference-image analysis, vertical profile generation, adaptive thresholding, mercury-level detection, temperature calibration, graphical display, and CSV logging.

The project successfully converts a visual thermometer-reading task into a digital processing pipeline. The use of a narrow ROI reduces unnecessary computation, while frame averaging and filtering improve the stability of the image. The adaptive threshold mechanism allows the detection threshold to respond to measured background and contrast conditions.

The prototype provides a foundation for further development toward a more robust automated meteorological observation system. Before claiming a specific measurement accuracy, however, the system should undergo controlled calibration and validation against a reference thermometer. Future work should also quantify error, repeatability, detection confidence, robustness to illumination changes, and long-term performance.

Overall, the training project provided practical experience at the intersection of embedded systems, computer vision, Python programming, instrumentation, and meteorological observation. It demonstrates how digital image processing can be applied to automate a conventional measurement task while retaining the original physical sensing instrument.

# REFERENCES

1. OpenCV Documentation — Image processing, filtering, thresholding and image comparison functions.

2. NumPy Documentation — Numerical array processing and statistical operations.

3. Picamera2 Documentation — Raspberry Pi camera interface and configuration.

4. Raspberry Pi Documentation — Raspberry Pi hardware and camera/software environment.

5. Python Documentation — Python standard library, threading, CSV, datetime and Tkinter.

6. India Meteorological Department (IMD) — Official meteorological organization information and services.

7. Project source code: final_stevenson_code.py — supplied implementation used as the primary technical basis of this report.

# APPENDIX A

# KEY CONFIGURATION PARAMETERS

| Parameter | Value |
| --- | --- |
| FRAME_W × FRAME_H | 700 × 640 |
| ROI_X, ROI_Y | 323, 1 |
| ROI_W × ROI_H | 9 × 620 |
| PIXEL_AT_5C | 505 |
| PIXEL_AT_45C | 63 |
| TEMP_LOW | 12 °C |
| TEMP_HIGH | 45 °C |
| Required consecutive rows | 7 |
| White LED GPIO | BCM 18 |
| CSV interval | 30 seconds |
| Reference file | REFERENCE.png |
| Log file | temperature_log.csv |